FOR FAX CLASS 2 OPERATION – ONLY IF YOUR FAX MODEM SUPPORTS CLASS 2

To determine what CLASS  your modem is, and if Zetafax will support it, use the reference on the Zetafax/Equisys support site.

It involves running Hyperterminal and typing (AT+FCLASS=?)

On the Zetafax Server Configuration Console, within the “Devices” Section:

Click on the “More” button to get to the Advanced Options

Double click on your fax device to get to the Device Options

UNCHECK the “Use Class 1 (if available)” to force Class 2 operation.  This will greatly speed up fax transmissions.  Now, I felt confident to do this with our fax modem hardware because it is a modern Class 2 device and has its own DSP controller right on the PCI card itself.  If you are using a “typical” inexpensive faxmodem that comes with a new computer, which DOES NOT have its own DSP controller (it uses a computer’s CPU as its DSP controller), you should test operation before using this setting in your production environment